Lab Practical #05:

Study the concept of VLAN using packet tracer.

Practical Assignment #05:

1. Implement the different network structures in VLAN and VLAN trunking. Also check connectivity between them using ping command or PDU utility.

Instructions:

- Different VLANs configuration setup screenshot. (VLAN example given by lab faculty)

- Write steps to create VLANs in packet tracer.

- Mention IP address of each pc as label.

- Ping command or PDU screenshot between two VLANs.

1.

VLAN_1:

1.

VLAN_1:

# 1.1 Steps to Create in Packet Tracer

## 1. Add Devices in Workspace

- Drag one 2960-24TT switch into workspace.

- Add 8 PCs → split into two groups:

  - BTECH Group (Left side) → PC0, PC1, PC2, PC3.

  - MCA Group (Right side) → PC4, PC5, PC6, PC7.

- Connect each PC to the switch using Straight-Through cables.

## 2. Assign IP Addresses to PCs

On each PC →Desktop → IP Configuration:

- BTECH VLAN(VLAN 10) o PC0 → 192.168.1.2 / 255.255.255.0 o PC1 → 192.168.1.3 / 255.255.255.0 o PC2 → 192.168.1.4 / 255.255.255.0 o PC3 → 192.168.1.5 / 255.255.255.0

- MCA VLAN (VLAN 20) o PC4 → 192.168.1.6 / 255.255.255.0 o PC5 → 192.168.1.7 / 255.255.255.0 o PC6 → 192.168.1.8 / 255.255.255.0 o PC7 → 192.168.1.9 / 255.255.255.0

Add labels to each PC to display its IP Address.

## 3. Create VLANs on Switch

Click on the switch → Config tab → VLAN Database:

 	VLAN 10 → Name: BTECH → Add.  	VLAN 20 → Name: MCA → Add.

## 4. Assign Switch Ports to VLANs

Go to Switch → Config tab → Interfaces.

 	Assign FastEthernet 0/1 – 0/4 → VLAN 10 (BTECH).  	Assign FastEthernet 0/5 – 0/8 → VLAN 20 (MCA).

This ensures each group of PCs belongs to its own VLAN.

## 5. Test VLAN Connectivity

- Open Command Prompt on a PC.

- Ping another PC in the same VLAN → should be successful.

- Ping a PC in a different VLAN → will fail (unless VLAN trunking or router is configured).

2.  VLAN_2:

# 2.1 Steps To Create VLAN

## 1. Add Devices in Workspace

- Drag and drop 3 switches (2960-24TT) into workspace.

- Add 9 PCs and arrange them into 3 groups:

o Engineering (3 PCs) o Marketing (3 PCs) o Accounting (3 PCs)

- Connect PCs to switches using Straight-Through cables.

- Connect switches together with crossover cables.

## 2. Assign IP Addresses to PCs

Configure each PC → Desktop → IP Configuration:

- Engineering VLAN (VLAN 10) o PC0 → 192.168.1.2 / 255.255.255.0 o PC3 → 192.168.1.5 / 255.255.255.0 o PC6 → 192.168.1.8 / 255.255.255.0

- Marketing VLAN (VLAN 20) o PC1 → 192.168.1.3 / 255.255.255.0 o PC4 → 192.168.1.6 / 255.255.255.0 o PC7 → 192.168.1.9 / 255.255.255.0

- Accounting VLAN (VLAN 30) o PC2 → 192.168.1.4 / 255.255.255.0 o PC5 → 192.168.1.7 / 255.255.255.0 o PC8 → 192.168.1.10 / 255.255.255.0

Add labels to each PC to display its IP Address.

## 3. Create VLANs on Each Switch

Click on each switch → Config tab → VLAN Database:

- VLAN 10 → Name: Engineering → Add.

- VLAN 20 → Name: Marketing → Add.

- VLAN 30 → Name: Accounting → Add.

Do this on Switch0, Switch1, and Switch2

## 4. Assign Ports to VLANs

For each switch → Config tab → select Interface (FastEthernet) → assign VLAN.

Example for Switch0:

- Port F0/1 → VLAN 10 (PC 192.168.1.2 – Engineering)

- Port F0/2 → VLAN 20 (PC 192.168.1.3 – Marketing)

- Port F0/3 → VLAN 30 (PC 192.168.1.4 – Accounting)

Similarly configure Switch1 and Switch2.

## 5. Configure Trunk Links Between Switches

For switch-to-switch connections:

- Select interface where switches are connected  	Set Mode → Trunk.

- This allows VLAN 10, 20, and 30 traffic to pass across switches.

Repeat for all inter-switch connections.

## 6.  Test VLAN Connectivity

- Open Command Prompt on a PC.

- Ping another PC in the same VLAN → should be successful.  	Ping a PC in a different VLAN → will fail.

3. VLAN_3:

# 3.1  Steps To Create VLAN

## 1.  Add Devices in Workspace

- Drag and drop 2 switches (2960/2950) into workspace.

- Add 8 PCs into the workspace.  	Divide them into 2 groups:

o VLAN 1 → 4 PCs o VLAN 2 → 4 PCs

- Connect PCs to switches using Straight-Through cables.

- Connect the two switches together using a Crossover cable.

## 2.  Assign IP Addresses to PCs

Go to each PC → Desktop → IP Configuration and assign IP addresses:

- VLAN 1 o PC0 → 192.168.1.2 / 255.255.255.0 o PC1 → 192.168.1.3 / 255.255.255.0 o PC2 → 192.168.1.6 / 255.255.255.0

o PC3 → 192.168.1.7 / 255.255.255.0

- VLAN 2 o PC4 → 192.168.1.4 / 255.255.255.0 o PC5 → 192.168.1.5 / 255.255.255.0 o PC6 → 192.168.1.8 / 255.255.255.0 o PC7 → 192.168.1.9 / 255.255.255.0

Add labels to each PC to display its IP address.

3.  Create VLANs on Each Switch For each switch → Config tab → VLAN Database:

- VLAN 10 → Name: VLAN_1 → Add.

- VLAN 20 → Name: VLAN_2 → Add.

Repeat on both Switches.

## 4.  Assign Ports to VLANs

For each switch → Config tab → select Interface (FastEthernet) → assign VLAN.

- Switch0 Example:

  - Port F0/1 → VLAN 1 (PC0 – 192.168.1.2) o Port F0/2 → VLAN 1 (PC1 – 192.168.1.3) o Port F0/3 → VLAN 2 (PC4 – 192.168.1.4) o Port F0/4 → VLAN 2 (PC5 – 192.168.1.5)

- Switch1 Example:

  - Port F0/1 → VLAN 1 (PC2 – 192.168.1.6) o Port F0/2 → VLAN 1 (PC3 – 192.168.1.7) o Port F0/3 → VLAN 2 (PC6 – 192.168.1.8) o Port F0/4 → VLAN 2 (PC7 – 192.168.1.9)

## 5. Configure Trunk Link Between Switches

- For the interface connecting Switch0 ↔ Switch1 → set to Trunk mode.

- This allows both VLAN 1 and VLAN 2 traffic to pass across switches.

## 6. Test VLAN Connectivity

- Open Command Prompt on a PC.

- Ping another PC in the same VLAN → should succeed.  	Ping a PC in a different VLAN → should fail.

4. VLAN_4:

# 4.1  Steps To Create VLAN

## 1.  Add Devices in Workspace

- Drag and drop 2 switches (2960-24TT) into the workspace.

- Add 8 PCs and arrange them into 2 groups:

o VLAN 1 group → 4 PCs o VLAN 2 group → 4 PCs

- Connect PCs to switches using Straight-Through cables.

- Connect Switch0 ↔ Switch1 using a Crossover cable.

## 2.  Assign IP Addresses to PCs

Configure each PC → Desktop → IP Configuration:

- VLAN 1 o PC0 → 192.168.1.2 / 255.255.255.0 o PC1 → 192.168.1.3 / 255.255.255.0 o PC6 → 192.168.1.8 / 255.255.255.0 o PC7 → 192.168.1.9 / 255.255.255.0

- VLAN 2 o PC2 → 192.168.1.6 / 255.255.255.0 o PC3 → 192.168.1.7 / 255.255.255.0 o PC4 → 192.168.1.4 / 255.255.255.0 o PC5 → 192.168.1.5 / 255.255.255.0

Add labels to each PC to display its IP Address.

3. Create VLANs on Each Switch Click on each switch → Config tab → VLAN Database:

- VLAN 1 → Name: VLAN_1 → Add.

- VLAN 2 → Name: VLAN_2 → Add.

Do this on both Switch0 and Switch1.

## 4. Assign Ports to VLANs

 	Switch0:

- Port F0/1 → VLAN 1 (PC0 – 192.168.1.2) o Port F0/2 → VLAN 1 (PC1 – 192.168.1.3) o Port F0/3 → VLAN 2 (PC2 – 192.168.1.6) o Port F0/4 → VLAN 2 (PC3 – 192.168.1.7)  	Switch1:

- Port F0/1 → VLAN 2 (PC4 – 192.168.1.4) o Port F0/2 → VLAN 2 (PC5 – 192.168.1.5) o Port F0/3 → VLAN 1 (PC6 – 192.168.1.8) o Port F0/4 → VLAN 1 (PC7 – 192.168.1.9)

## 5. Configure Trunk Link Between Switches

For the interface connecting Switch0 ↔ Switch1:

- Select the interface (e.g., FastEthernet 0/24).

- Set Mode → Trunk.

- This allows VLAN 1 & VLAN 2 traffic to pass across switches.

## 6. Test VLAN Connectivity

- Open Command Prompt on a PC.

- Ping another PC in the same VLAN → should be successful.  	Ping a PC in a different VLAN → will fail.